Turn on Radio by turning the short knob on its top until it clicks.

Setting radio channel.  Radio has a primary channel (large number) and a secondary channel (small number in upper right).  Both have to match on each radio you want to talk to each other.

- Push the menu button down for 1 second until the large number starts flashing.  Change the number up or down by pressing the plus and minus buttons.

- Push the menu button a second time and the small number will flash.  Change this number the same way.

- Push the menu button a third time and you will begin scrolling through other options.  Keep pushing the menu button until you return to the screen with the large and small numbers you selected (4 pushes).

- Lock the channels into place.

- Test and confirm that radios are talking to each other before you deploy.

Lock and unlock the channels by holding down the “Menu” button for about 5 seconds until you hear it beep.  A small lock icon will appear and disappear in the middle top of the display.

Volume is also controlled by this knob.

Turn off by turning knob until it clicks.

Alert Feature.  The musical note button sends out an alert tone to all radios on the same channel.  It is useful if you are trying to get everyone’s attention.

Remove belt clip by pressing down on the portion of the clip with the “M” on it and pulling back.  Clip is replaced in the same manor.

Replace batteries (you can do this with the belt clip in place by turning it to the side):

Slide your fingernail under the recessed latch on the lower back plate of the radio and push up.  Lift the lower part of the back plate until it clears the rest of the radio and pull down.  (Plate has stationary “pins” that insert into the top of the back).

Remove batteries by holding the radio face up and gently striking the bottom of the radio against the heel of your hand.  The batteries will drop out.

Insert batteries, according to diagram inside the radio (for AA) or on the battery itself (rechargeable).  Place the top end of the battery in first then snap them down and into place.

Replace the back plate by inserting the stationary “pins” into their holes in the top back of the radio, then snapping the bottom into place.

Battery life is monitored on the top left side of the display.

Signing out radios.  Each radio has a unique number printed on the back.  Please log the name of each person and the radio assigned to them.  It’s a lot easier to find missing radios later!

# Communications Plan

Set up a communication plan so all team members know what they are expected to do.  Issues to discuss include:

- Team name designation (Team 1, Team 500/John, SR1, etc)

- Primary and backup radio frequencies (if using radios)

- Emergency recall (in case the safety officer detects a hazardous condition that could endanger the search team) – Use whistle if not using radios.

- Status check (to confirm the team is OK and let them know how much time has elapsed*)

- Recall procedure (for rest or to send in fresh people)

# Effective Messages

Keep radio communication to a minimum - If the person you need to talk to is close by, communicate face to face.

## Remember to Key – Hesitate – Talk   Hold the side PTT (Push to talk) button down in the middle.  Wait 2 seconds and then talk.

Keep it Simple - the more complex a message, the more likely it will be misunderstood.

Example:  "This is Mike, Search & Rescue team leader. We are at the building at 123 Main St. where the emergency involving possible people hurt inside the house that was just reported is being taken care of by my team.  We have commenced operations by locating our triage tags in our trauma bags and have begun triaging and moving the victims.”

Could be shortened to:  “Command Post, this is the Search & Rescue team leader. My team is on scene at 123 Main St. We are setting up triage. Over."

If the Command Post needs more information, they will ask.

Each message should contain information in the following order:

- Title of Receiver - "example: Command Post"

- Title of Sender -  “example:  S&R Team Leader” (If you have Emergency Traffic, it should be indicated in this initial broadcast). “This is Search & Rescue Team Leader with Emergency Traffic”

- Wait for Receiver to acknowledge you.  Do not continue the transmission until you are sure the Receiver is ready to receive.

- Your Location

- Message

- Termination  – “Over” (“over” is the universal term for ending a transmission.  Others will know that the air is clear for their transmission.)

- Receiver will acknowledge receipt.  Receiver may chose to repeat the message to confirm it was correctly understood.

8.  When you have finished your FINAL transmission of the conversation, say   “Out” or “Clear”.  Hold in the “talk” button for a second after you finish speaking so your last word is not cut off.

NOTE:  If you are told that the first part of your transmission is being cut off, it’s because you’re not waiting long enough after you push down the PTT or you’re not pushing down hard enough.

Sample Communication:

“Command Post, this is S&R Team leader with Emergency Traffic”

“S& R Team leader, this is Command Post, go ahead, over"

“Command Post, S&R Team is on scene at 123 Main St.  We need the First Aid Team for injuries, over.”

“Command Post copies, need the First Aid Team to 123 Main St.  Command Post Out”

Use Plain English – Most emergency responders have changed from using codes to “clear text” (plain English) to avoid misunderstandings.

Listen before talking.  Be sure no one else it talking before you begin.

After depressing the talk button, wait for two seconds before speaking.

Speak Slowly.  Be sensitive to the amount of information and detail the person on the other end needs to write down.

Avoid broadcasting deaths over the air.  You never know who is listening! – Use a messenger.

Avoid broadcasting children’s names over the air (if the information about them is sensitive) – Use a messenger

###### Using Whistles to Communicate

Emergency Recall

- Three short blows by safety officer means the search team needs to exit right away.  Search team acknowledges by repeating the signal.

- Three short blows by the search team means they are in trouble and request help.

Status Check

- One long blow by the safety officer means that the welfare of the team is being checked. Search team acknowledges by repeating the signal.

Recall Procedure

- Three long blows means the search time has expired and the search team should exit the building.  Search team acknowledges by repeating the signal.

*Search times should be between 10 and 30 minutes, depending upon the situation…shorter time periods for more stressful/hazardous situations.